# Auris COM Simulation

# Testing Procedure

Andrew S Johnson
ansjohns@mtu.edu

Michigan Technological University

Aerospace Enterprise

1400 Townsend Dr.

Houghton, MI 49931

| Author : | Author Signature |
| --- | --- |
| Team Leader : | Team Leaders Signature |
| Systems Engineer : | Systems Engineers Signature |
| Project Manager : | PM Signature if needed |
| Chief Engineer : | CE Signature if needed |
| Principle Investigator : | PI Signature if needed |

Revision History

| Revision | Revised By | Revised On | Changes |
| --- | --- | --- | --- |
| 1 | A Johnson | 04 08 2018 | Initial writing |
| 2 | A Johnson | 04 10 2018 | Insert MTU Access Calculation |

List of Needed Components/Equipment

| Owner (Subteam) | Components/Equipment | Quantity | Reason | See Pages |
| --- | --- | --- | --- | --- |
| ME | Computer w/ STK | 1 | To run Simulation | 1 |

Table of Contents

Requirement Verification…………………………………………………………………………………2

List of Needed Components/Equipment…………………………………………………………….….3

Table of Contents…………………………………………………………………………………………4

1	Test Overview	5

1.1	Objectives	5

1.2	Test Schedule	5

2	Procedure	5

2.1	Assumptions	5

2.1.1	Assumption 1	5

2.1.2	Assumption 2	5

2.2	Preparation	5

2.2.1	Prep Step 1	6

2.2.2	Prep Step 2	6

2.2.3	Prep Step 3	6

2.2.4	Prep Step 4	6

2.2.5	Prep Step 5	7

2.2.6	Prep Step 6	7

2.3	Test Environment	8

2.4	Test Execution	8

	Major step 1	8

2.4.1	Major Step 2	8

3	Results	9

Appendix A 	Test Logs………………………………………………………………………………11

List of Figures

No table of figures entries found.

List of Tables

No table of figures entries found.

# Test Overview

The purpose of this test is to get an idea of the expected length of communications access windows between the ground station and the spacecraft.

## Objectives

This test will provide reference material to several sub-teams concerned with communicating data to and from the spacecraft in order to complete the mission (MOP, CDH, PYL, COM).  This test is only a simulation and can not be used to verify any requirements.

## Test Schedule

- Set and verify mission and environmental parameters in the scenario.

- Compute access between the ground station and the spacecraft (Upstream and downstream)

- Report results from the test.

# Procedure

## Assumptions

This test makes various assumptions about environmental conditions and communications system performance that may vary from the real world.  It is assumed that the tester has a solid working knowledge of the STK software and how to accurately model the components of the communications system.

### Assumption 1

It is assumed that an up-to-date (as of this writing) mission scenario exists with all objects necessary to perform an analysis on the communications system already existing.

### Assumption 2

Due to the nature of the simulation software, it is assumed that a communications link will be established with the ground station the moment the spacecraft is within range. (Line of sight)

## Preparation

This test requires several environmental models that need to be enabled before the analysis is run.  These models take into account RF signal propagation, attenuation from rain, clouds, fog, atmosphere, tropospheric scintillation and ionospheric fading.  These models will be configured in the RF Environment section of the scenario properties window.

### Prep Step 1

In the Environmental Data tab, select “Use ITU-R P.618 Section 2.5” under “Total Propagation Calculation”. [1]

### Prep Step 2

In the “Rain & Cloud & Fog” tab, select “Use” under both the Rain Model section and the Clouds and Fog Model section. The Rain model to be used is ITU-R P618-12, Surface Temperature is set to 15.55 °C and “Enable Cross Polarization Loss” is checked.

ITU-R P840-6 is selected for the cloud and fog model with a cloud ceiling of 3km, a cloud layer thickness of 0.5km and a cloud temperature of 0°C.  Liquid Water Content Density Value is set to 7.5 g/m3 (standard ground-level surface water vapor density) [2].

### Prep Step 3

In the Atmospheric Absorption tab, “Use” is unchecked because the models are valid for a frequency range of 1 GHz to > 350 GHz.  Auris will be communicating in the 450 MHz band.

### Prep Step 4

In the Tropo Scintillation tab, Use is enabled with ITU-R P618-12 set as the model.  Compute deep fade is enabled with Surface temperature set to 15.55 °C, tropo fade outage a 0.1%, Percent Time Refractivity Gradient < -100N units/km set to 10%.  Fade depth for the average year is selected.

### Prep Step 5

In the Iono Fading tab, Use model ITU-R P531-13 is enabled. [3]

### Prep Step 6

The components of the communications system are defined as indicated in the following tables.

#### Ground station receiver

| Parameter | Value | Unit |
| --- | --- | --- |
| Frequency | 455 | MHz |
| Antenna to LNA Line Loss | 0 | dB |
| LNA Gain | 0 | dB |
| LNA to Receiver Line Loss | 0 | dB |
| Rain Model Outage Percent | 0.1 | Percent |
| Link Margin C/N | 7 | dB |
| Noise Temperature | 290 | Kelvin |
| Demodulator | MSK | - |

#### Ground station transmitter

| Parameter | Value | Unit |
| --- | --- | --- |
| Frequency | 455 | MHz |
| Power | 50 | W |
| Data Rate | 9600 | bps |
| Modulation | MSK | - |
| Signal Bandwidth | 148.8 | kHz |

#### Ground station antenna

| Parameter | Value | Unit |
| --- | --- | --- |
| Type | Helix | - |
| Design Frequency | 455 | MHz |
| Diameter | 0.339 | m |
| Efficiency | 55 | Percent |
| Turn Spacing | 0.08 | M |
| Number of Turns | 28 | - |
| Back-lobe Gain | -30 | dB |

#### Space station receiver

| Parameter | Value | Unit |
| --- | --- | --- |
| Frequency | 455 | MHz |
| Antenna to LNA Line Loss | 0 | dB |
| LNA Gain | 0 | dB |
| LNA to Receiver Line Loss | 0 | Db |
| Rain Model Outage Percent | 0.1 | Percent |
| Link Margin C/N | 27.1 | dB |
| Noise Temperature | 581.2 | Kelvin |
| Demodulator | MSK | - |
| Receiver Bandwidth | 25 | kHz |

#### Space station transmitter

| Parameter | Value | Unit |
| --- | --- | --- |
| Frequency | 455 | MHz |
| Power | 3 | W |
| Data Rate | 9600 | bps |
| Modulation | MSK | - |
| Signal Bandwidth | 52.8 | kHz |

#### Space station antenna

| Parameter | Value | Unit |
| --- | --- | --- |
| Type | Isotropic | - |
| Design Frequency | 455 | MHz |
| Main-lobe Gain | 0 | dB |
| Efficiency | 100 | Percent |

## Test Environment

The test can be performed on any computer with a working copy of STK.

## Test Execution

### Major step 1

Using the access tool an access is calculated FROM the ground station receiver TO the space station transmitter.

The time-period for the access analysis is 14 Apr 2018 16:00:00.000 UTCG to 21 Apr 2018 16:00:00.000 UTCG.

Click “Compute”.

### Major Step 2

At the completion of the analysis, open the access report.  Save as a txt file or csv file.

MTU_Rx1-To-AurisCom_Tx

----------------------

Access        Start Time (UTCG)           Stop Time (UTCG)        Duration (min)

------    ------------------------    ------------------------    --------------

1    14 Apr 2018 16:40:09.797    14 Apr 2018 16:44:11.096             4.022

2    14 Apr 2018 18:12:17.872    14 Apr 2018 18:18:31.756             6.231

3    14 Apr 2018 18:18:35.031    14 Apr 2018 18:21:26.561             2.859

4    14 Apr 2018 19:49:02.599    14 Apr 2018 19:58:15.363             9.213

5    14 Apr 2018 21:28:41.194    14 Apr 2018 21:34:59.019             6.297

6    14 Apr 2018 23:06:57.122    14 Apr 2018 23:13:26.447             6.489

7    15 Apr 2018 00:44:26.823    15 Apr 2018 00:50:01.898             5.585

8    15 Apr 2018 02:18:58.924    15 Apr 2018 02:28:58.824             9.998

9    15 Apr 2018 03:57:04.393    15 Apr 2018 04:01:41.968             4.626

10    15 Apr 2018 16:04:52.872    15 Apr 2018 16:06:59.922             2.118

11    15 Apr 2018 17:38:47.952    15 Apr 2018 17:42:00.277             3.205

12    15 Apr 2018 17:42:04.152    15 Apr 2018 17:45:03.121             2.983

13    15 Apr 2018 19:12:49.246    15 Apr 2018 19:16:44.796             3.926

14    15 Apr 2018 19:16:56.546    15 Apr 2018 19:20:59.734             4.053

15    15 Apr 2018 20:51:31.932    15 Apr 2018 20:57:01.215             5.488

16    15 Apr 2018 22:30:34.455    15 Apr 2018 22:35:27.477             4.884

17    16 Apr 2018 00:06:39.410    16 Apr 2018 00:08:41.210             2.030

18    16 Apr 2018 00:08:57.335    16 Apr 2018 00:11:42.660             2.755

19    16 Apr 2018 00:11:57.410    16 Apr 2018 00:15:33.085             3.595

20    16 Apr 2018 01:43:08.055    16 Apr 2018 01:52:08.920             9.014

21    16 Apr 2018 03:19:40.152    16 Apr 2018 03:24:23.810             4.728

22    16 Apr 2018 17:00:44.388    16 Apr 2018 17:03:08.838             2.408

23    16 Apr 2018 17:05:08.388    16 Apr 2018 17:08:34.535             3.436

24    16 Apr 2018 18:36:24.414    16 Apr 2018 18:39:02.089             2.628

25    16 Apr 2018 18:39:06.639    16 Apr 2018 18:41:37.489             2.514

26    16 Apr 2018 18:41:42.589    16 Apr 2018 18:44:46.596             3.067

27    16 Apr 2018 20:14:10.601    16 Apr 2018 20:20:39.218             6.477

28    16 Apr 2018 21:54:03.758    16 Apr 2018 21:58:19.345             4.260

29    16 Apr 2018 23:30:25.182    16 Apr 2018 23:38:40.982             8.263

30    17 Apr 2018 01:07:36.486    17 Apr 2018 01:14:08.911             6.540

31    17 Apr 2018 02:43:02.427    17 Apr 2018 02:49:40.705             6.638

32    17 Apr 2018 16:24:45.350    17 Apr 2018 16:32:05.143             7.330

33    17 Apr 2018 17:59:55.821    17 Apr 2018 18:02:09.521             2.228

34    17 Apr 2018 18:02:12.621    17 Apr 2018 18:05:39.871             3.454

35    17 Apr 2018 18:05:43.771    17 Apr 2018 18:08:34.620             2.847

36    17 Apr 2018 19:36:54.565    17 Apr 2018 19:44:23.224             7.478

37    17 Apr 2018 21:17:30.250    17 Apr 2018 21:21:14.944             3.745

38    17 Apr 2018 22:54:05.926    17 Apr 2018 23:01:39.571             7.561

39    18 Apr 2018 00:31:43.902    18 Apr 2018 00:36:55.952             5.201

40    18 Apr 2018 00:37:01.152    18 Apr 2018 00:39:50.141             2.816

41    18 Apr 2018 02:07:31.500    18 Apr 2018 02:13:39.500             6.133

42    18 Apr 2018 15:48:52.611    18 Apr 2018 15:55:29.375             6.613

43    18 Apr 2018 17:23:30.163    18 Apr 2018 17:25:33.513             2.056

44    18 Apr 2018 17:25:36.063    18 Apr 2018 17:29:28.463             3.873

45    18 Apr 2018 17:29:32.013    18 Apr 2018 17:32:16.396             2.740

46    18 Apr 2018 19:00:08.507    18 Apr 2018 19:08:10.887             8.040

47    18 Apr 2018 20:40:27.486    18 Apr 2018 20:44:29.787             4.038

48    18 Apr 2018 22:17:47.308    18 Apr 2018 22:24:31.556             6.737

49    18 Apr 2018 23:53:39.063    18 Apr 2018 23:55:44.463             2.090

50    18 Apr 2018 23:55:52.663    18 Apr 2018 23:59:48.663             3.933

51    18 Apr 2018 23:59:53.363    19 Apr 2018 00:03:22.863             3.492

52    19 Apr 2018 01:31:49.792    19 Apr 2018 01:36:38.142             4.806

Global Statistics

-----------------

Min Duration          17    16 Apr 2018 00:06:39.410    16 Apr 2018 00:08:41.210             2.030

Max Duration           8    15 Apr 2018 02:18:58.924    15 Apr 2018 02:28:58.824             9.998

Mean Duration                                                                                4.612

Total Duration                                                                             350.538

Results

14 Apr 2018 16:12:11

Approved AGI Educational Alliance Partner

Facility-Houghton_GS-Receiver-MTU_Rx1-To-Satellite-Auris-Transmitter-AurisCom_Tx:  Link Budget - Short Form

Time (UTCG)         Rcvd. Frequency (MHz)   Rcvd. Iso. Power (dBW)    C/N (dB)  Eb/No (dB)        BER

------------------------   ---------------------   ----------------------   --------   ----------   ------------

14 Apr 2018 16:40:09.797              455.001653               -176.902      4.0240       11.4276   6.782288e-08

14 Apr 2018 16:41:09.000              454.999173               -163.349     17.8195       25.2231   1.000000e-30

14 Apr 2018 16:42:09.000              454.996786               -161.286     19.8191       27.2227   1.000000e-30

14 Apr 2018 16:43:09.000              454.994830               -161.465     19.8017       27.2053   1.000000e-30

14 Apr 2018 16:44:09.000              454.993383               -170.393     10.8688       18.2724   1.000000e-30

14 Apr 2018 16:44:11.096              454.993341               -171.468      9.7937       17.1973   6.435405e-25

Appendix A: Test NOTES

ITU models are only valid and can only be computed through December of 2018, so analysis using those models was performed for the current year.  It is assumed that there will be no shattering changes in the laws of physics between now and the time the mission will take place and so it can be assumed that results from this analysis will be typical of what can be expected at the time of the mission.

# References

| [1] | "Recommendation ITU-R P.618-13 Propagation data and prection methods required for the design of Earth-space telecommunication systems," International Telecommunication Union, Geneva, 2017. |
| --- | --- |
| [2] | "Recommendation ITU-R P.676-11 Attenuation by atmospheric gases," International Telecommunication Union, Geneva, 2016. |
| [3] | "Recommendation ITU-R P.531-13 Ionospheric propagation data and prediction methods required for the design of satellite services and systems," International Telecommunication Union, Geneva, 2016. |